# Deflection

- This section analyzes the structural rigidity of the turntable design to ensure it meets the specified deflection limits under static and dynamic loading conditions.

- Design Configuration: The turntable floor is supported by a structure where the main load-bearing beams are configured as Fixed-Fixed Beams. The beams are rigidly connected to the central hub and the outer rim (which is supported by the bearing/roller system).

- Beam Cross-Section: Rectangular, 800  mm  × 300 mm.

- Material: Aluminum 7075-T6.

  - Young's Modulus (E ): 71.7 GPa.

2. Vertical Deflection Analysis (Static)

2.1 Requirement

- Max Vertical Deflection: < 3.0  mm.

- Max Relative Displacement (Difference): < 500 μm  (0.5 mm).

2.2 Methodology

The vertical deflection at the mounting points (A, B, C) is calculated using the standard formula for a Fixed-Fixed beam with a concentrated load (weight of the payload) applied at distance a from the center:

δz = Pa3b23EIL3, a<b

δz= M0a2b2a-b3EIL3 a>b

Where:

- $P = m⋅g (Gravitational Force).

- Ix = bh312 = 0.8⋅0.3312 = 1.79 ×10-3 m4   (Moment of Inertia about the horizontal axis).

- L = 4.6  m (Beam span/Radius).

2.3 Result

| Point | Mass (kg) | Radius a (m) | Calculated Deflection δz​ (mm) |
| --- | --- | --- | --- |
| A | 3,000 | 3.6055 | 0.018 |
| B | 5,000 | 4.12 | 0.009516 |
| C | 1,500 | 2.00 | 0.0212 |

### 2.4 Relative Displacement Check

The maximum difference in vertical displacement between any two points is calculated:

- |δA -δB|  = |0.018 - 0.00516| = 0.002373 mm

- |δA -δC|  = |0.018 - 0.0212| = 0.00922 mm

- |δB -δC|  = |0.00516 - 0.0212| = 0.01599 mm

Result: The maximum relative displacement is 16 μm, which is significantly below the limit of 500μm.

## 3. Horizontal Deflection Analysis (Dynamic)

### 3.1 Requirement

- Max Horizontal Deflection: The center of the mounting point must not deflect outside a circle of diameter 2 mm (Radius < 1.0  mm).

### 3.2 Methodology (Worst-Case Scenario)

We analyze the deflection during the maximum acceleration phase at full speed (15  rpm). This generates two horizontal forces:

- Centrifugal Force (Fr): Radial outward force.

- Tangential Inertia Force (Ft): Perpendicular to the radius, caused by angular acceleration (α).

Using Point A as the reference example:

- Total Horizontal Force (Ftotal): ≈26779.99N

- Moment of Inertia (Iy)Resisting horizontal bending, Iy= h⋅b312=1.28×10-2 m4

3.3 Calculation

Using the fixed-fixed beam model for horizontal load:

δh = M0L12EI

3.4 Vertical Deflection Induced by Moment (Coupling Effect)

Since the mass is located at z=0.5  m (above the beam), the horizontal forces create an overturning moment (M0) that twists the beam vertically.

- Moment M0≈13,390  Nm.

3.5 Result

| Point | Mass (kg) | Radius a (m) | Calculated Deflection δh​ (mm) |
| --- | --- | --- | --- |
| A | 3,000 | 3.6055 | 0.043699 |
| B | 5,000 | 4.12 | 0.0095242 |
| C | 1,500 | 2.00 | 0.00154 |

Result: The maximum relative displacement is 16 μm, which is significantly below the limit of 500μm.

Summary: The proposed design using 800mm x 300mm Aluminum 7075 beams in a Fixed-Fixed configuration provides exceptional rigidity. The calculated deflections are orders of magnitude smaller than the allowable limits, ensuring stable operation for the scientific instrumentation payload.