# Teaching Pendant script

Introduction:

Background :

Last year, Professor Zhang Xu, an academician of the Chinese Academy of Sciences, remotely controlled a surgical robot deployed at Beijing to successfully perform a radical prostatectomy on a patient from Rome, Italy.

Modern surgical robots (such as the da Vinci system) achieve positional tracking mapping: the surgeon performs extremely subtle movements on the master arm, while the slave arm moves synchronously within the patient's body in real time. This control mode not only enables minimally invasive procedures but also filters out natural hand tremors, greatly improving precision.

Before surgery, traditional surgery requires making a large incision on the patient. Also during the surgery, it relies on surgeon’s hand stability. After surgery it will face intense pain and slow recovery. In comparison, robotic surgery just needs tiny incisions and the system will filter the shaking and after surgery, patients will recover quickly.

Motivation:

The core motivation behind my project is to replicate this fundamental logic of 'master-slave control'. Although this is a small-scale demo, its working goal is the same as that of expensive surgical robots—to achieve highly intuitive, low-latency position-following mapping, allowing operators to break free from code and complex button presses and achieve precise control through the most natural body movements.

Demo:

The project aims to design a controller that is not only compact in size but also perfectly maps the degrees of freedom of the robotic arm. As demonstrated, every joint movement of the master arm is reflected in real time on the slave robotic arm.

Goal:

- 5 DOF robotic arm

- Synchronous motion mapping

- Less shaking

System overview:

In system, I adopted a typical master-slave topology. On the hardware side, I used four potentiometers as angle sensors. A highlight here is the introduction of the PCA9685 servo driver board, which communicates with the Arduino via the I2C bus. This not only solves the problem of insufficient Arduino PWM pins but also ensures stable current for multiple servos under heavy loads through an external power supply , preventing MCU resets. In terms of software logic, I wrote a general-purpose `moveMotor` function. It reads the analog signals from A0 to A3 in real time and precisely maps them to a pulse width of 650 to 2350 microseconds, thus achieving high-precision tracking of the slave arm's position to the teach pendant. As for the end effector, I designed an interactive logic: by reading the level of the D8 pin, I implemented one-button gripping and releasing control of the gripper.

Working principle of potentiometer:

First, let’s remove the potentiometer cover to see the inner structure as well. A potentiometer has three terminals, among two are fixed, and one is variable. Two fix outer terminals are connected to both ends of the resistive element formed into an arc of circle responsible for establishing an electrical resistance.

The variable middle terminal is connected to a sliding contact, often referred to as a wiper, attached with a rotary knob. Whenever er turn the knob, the wiper will run across the resistive element.

If only a middle terminal and one of the outer terminals of the potentiometer are used, it will act as a variable resistor. In this example, we use the potentiometer for brightness control. We connect the wiper to the positive terminal of the power supply and one end of the resistive element to the positive side of the LED. The amount of current flowing through the LED determines its brightness. If we turn the potentiometer in a way that increase the resistance, the current flow decreased, thus the LED become dimmer.

Working principle of servo motor:

Servos have three wires coming out of them. Out of which two will be used for Supply (positive and negative), and one will be used for the signal that is to be sent from the MCU.

So how do we use the potentiometer to control the motor manually. The signal enters the Servo’s circuit board and converted to a voltage. It passed through a comparator and then to a motor driver. The motor driver controls the rotation of the DC motor. The rotation causes the gears to rotate, which causes the final gear and servo arm to also rotate. The potentiometer connected to the final gear, then connected to the comparator in the internal circuit circuit board. When the knob is turned, the voltage changes between a minimum and maximum value. The comparator is going to compare the voltage of the potentiometer to the voltage of the controller signal. If there’s a difference, the motor will turn until the difference is close to zero. Then, the servo knows it is in the correct position. So it will wait there until another change.

Problems:

During development, I encountered a problem where signal noise caused the servo motor to vibrate. Also Mechanical structures have physical limits, but the original code mapping range exceeds the physical range. When a servo motor attempts to reach a position it cannot reach, it will stall, causing a surge in current that can easily burn out the servo controller or cause gear damage. Polarity Mismatch: * The installation orientation of the potentiometer may cause the signal to be opposite to the servo motor's action (e.g., the robotic arm moves to the right when turning left). Manually adjusting the hardware polarity is very time-consuming.

Something new

First, there's signal noise reduction. To address the inherent shaking of analog signals, I wrote an EMA filter. By weighting historical data, it makes the robotic arm's movements as smooth as silk, eliminating the 'jitter' phenomenon during servo idling.

Second, there's hardware protection. I realized that forcibly restraining the servo through physical structures is extremely dangerous. Therefore, I defined 'software no-go zones' in the code, limiting the angle output to ensure the servo never stalls and burns out.

Finally, I added a rapid calibration system. In complex mechanical assemblies, the potentiometer direction often mismatches with the servo's rotation direction. My system can reverse the signal polarity with a single click. This not only reduces debugging time by several times but also makes the entire system deployment more flexible.

Code:

Signal Stability: I implemented an EMA (Exponential Moving Average) Filter. By blending new raw data with historical values via the alpha coefficient, we successfully eliminated the high-frequency jitter common in analog potentiometers, resulting in silk-smooth motion.

Hardware Safety: To prevent the 'Singularity' issue where the robot hits a physical wall, I added Software Constraints using the constrain function. This ensures the motors never exceed their mechanical limits, effectively preventing motor stall and burnout.

Rapid Calibration: Finally, the moveMotorOptimized function includes a Software Polarity Toggle. This allows us to instantly reverse joint directions in the code without needing to re-wire the hardware, simplely adjust the true or false.

Future work:

Wireless Communication: By introducing ESP-NOW or 5G modules, cables can be completely eliminated, enabling true remote teaching.

Haptic Feedback: A vibration motor is added to the teach pendant so that the operator can feel the feedback when the robotic arm encounters an obstacle (similar to the feel of a surgical robot).

After learning the introduction to robotics:

Inverse Kinematics: Optimized algorithms that enable control to go beyond joint mapping and achieve precise synchronization of end-effector coordinates.